|  content-independent  |  content-dependent

Sampling Scheme                |       sampling        |      sampling

--------------------------------+-----------------------+--------------------

systematic sampling:           |                       |

count-based                    |           X           |

--------------------------------+-----------------------+--------------------

systematic sampling:           |                       |

time-based                     |           X           |

--------------------------------+-----------------------+--------------------

random sampling:               |                       |

n-out-of-N                     |           X           |

--------------------------------+-----------------------+--------------------

random, probabilitic sampling: |                       |

uniform probabilistic          |           X           |

--------------------------------+-----------------------+--------------------

random, probabilitic sampling: |                       |

non-uniform probabilistic      |                       |          X

--------------------------------+-----------------------+--------------------

random, probabilitic sampling: |                       |

non-uniform flow-state         |                       |          X

--------------------------------+-----------------------+---------------------

Scheme                        |    input parameters     |     functions

--------------------------------+-------------------------+---------------------

systematic sampling:           |     packet position     |  packet counter

count-based                    |     sampling pattern    |

--------------------------------+-------------------------+---------------------

systematic sampling:           |       arrival time      |  clock or timer

time-based                     |     sampling pattern    |

--------------------------------+-------------------------+---------------------

random sampling:               |   packet position       |  packet counter,

n-out-of-N                     |   sampling pattern      |  random numbers

|  (random number list)   |

--------------------------------+-------------------------+---------------------

random, probabilitic sampling: |        sampling         |  random function

uniform probabilistic          |       probability       |

--------------------------------+-------------------------+---------------------

random, probabilitic sampling: |  e.g. packet position   | selection function

non-uniform probabilistic      | or packet content(parts)|  probability calc.

--------------------------------+-------------------------+---------------------

random, probabilitic sampling: |    e.g. flow state      |  selection function,

non-uniform flow-state         | or packet content(parts)|  probability calc.

--------------------------------+-------------------------+---------------------

filtering: mask/match          |  packet content(parts)  |  filter function

--------------------------------+-------------------------+---------------------

filtering: hash-based          |   packet content(parts) |  hash function

--------------------------------+-------------------------+---------------------

filtering: router state        |     router state        |   router state

|                         |   discovery

--------------------------------+-------------------------+---------------------